Table of Contents

# PERSON A — Hardware / Chip Design / FPGA

Role: own everything between "a hex file exists" and "the FPGA produces the right number". RTL, memory architecture, clocking, constraints, timing closure, bitstream, on-board debug.

Mission: an NPU in Verilog on the Basys3 (XC7A35T) that takes a 50×50 grayscale image over UART and returns a digit class, with every intermediate number byte-identical to Person B's integer model.

"Done" for this role = a bitstream that (a) passes 100% of the frozen test-vector regression in simulation, (b) closes timing with positive WNS at the design clock, (c) classifies 20/20 of Person B's hand-drawn digits on real hardware with the same result as the golden model, (d) reports its own utilisation / latency / baud numbers into docs/ that Person B can cite.

Companion document: TEAM-B-software-ai.md. §0 (the contract) is identical in both files.

## §0 SHARED CONTRACT v1.0 — FROZEN. Do not change unilaterally.

This is the interface between the two halves of the project. It is the only thing that keeps two people working independently from producing two incompatible halves. Any change requires both people to agree, bumps the version, and means Person B re-exports vectors and Person A re-runs the full regression.

### 0.1 Numeric spec

| Item | v1 (MLP, first light) | v2 (CNN, the real NPU) |
| --- | --- | --- |
| Input image | uint8, 28×28, row-major, 255 = ink, 0 = background | uint8, 50×50, same convention |
| Activations | uint8, zero-point 0, per-tensor power-of-two scale 2^-n | same |
| Weights | int8, per-tensor scale 2^-m | int8, per-tensor (per-channel from v2.1) |
| Bias | int32, pre-scaled by the exporter into accumulator units | same |
| Accumulator | int32 | int32 (≥24 bits guaranteed used) |
| Requantisation | sat_u8(round_half_away_from_zero(acc >> s)), s integer ≥ 0 | same |
| Multiplier M | always 1 (power-of-two scales only — requant is a shift) | same |
| Activation fn | ReLU (negative → 0) then saturate to uint8 | same |
| Pooling | n/a | max, 2×2 stride 2, valid (no padding) |
| Padding | none (valid) | none (valid) |
| Argmax | lowest index wins on tie | same |
| Convolution order | n/a | [Cout][Cin][KH][KW] row-major |

Rule that matters most: rounding is round-half-away-from-zero, not floor, not banker's. Both sides implement the same three lines and both have a unit test with negative and tie values.

### 0.2 Model v1 (first light) and v2 (target)

v1: 784 -> FC 64 -> ReLU -> FC 32 -> ReLU -> FC 10 -> argmax        (28x28 input)
v2: conv 3x3 (1->8) -> ReLU -> maxpool 2x2
    -> conv 3x3 (8->16) -> ReLU -> maxpool 2x2
    -> FC (1936->10) -> argmax                                      (50x50 input)

### 0.3 File formats

- rtl/hex/w<N>.hex — one byte per line, lowercase two hex digits, $readmemh-compatible. FC: [out][in] row-major. Conv: [Cout][Cin][KH][KW].

- rtl/hex/b<N>.hex — int32 biases, one 8-digit hex word per line.

- model.json — contract version, model name/id, layer list with dimensions, bit widths, scales as exponents, and the SHA-256 of every .hex file. Person A's RTL and Person B's exporter both read it. If the SHA doesn't match what's in BRAM, the run is invalid.

### 0.4 Test vectors (the artifact that lets Person A work without Person B)

vectors/
  manifest.json        {"contract":"1.0","model":"mlp_v1","cases":[
                          {"id":0,"in":"case_0000.in.bin","dumps":["case_0000.l0.bin",...],
                           "logits":"case_0000.logits.txt","argmax":7}, ...]}
  case_0000.in.bin     uint8 input, row-major, 784 (v1) or 2500 (v2) bytes
  case_0000.l0.bin     activation dump after layer 0, uint8 row-major
  case_0000.logits.txt one int32 per line (10 lines) + final line "argmax <d>"

### 0.5 Host ↔ device interface (UART)

- Baud: 1,000,000 target (115200 for bring-up). 8N1. Byte-level, no flow control.

- Frame: 0xA5 | OP | LEN_LO | LEN_HI | PAYLOAD[LEN] | CRC8 | 0x5A (LEN little-endian, 16-bit).

- CRC8: poly 0x07, init 0x00, no reflection, no final XOR — over OP..PAYLOAD only.

- Response: 0x5A | (OP|0x80) | LEN_LO | LEN_HI | PAYLOAD | CRC8 | 0xA5, same CRC definition.

| OP | Name | Payload |
| --- | --- | --- |
| 0x01 | LOAD_IMG | raw uint8 image, row-major |
| 0x02 | LOAD_WEIGHTS | `layer_id(1) |
| 0x03 | START | empty |
| 0x04 | GET_RESULT | empty → returns `digit(1) |
| 0x05 | LOOPBACK | echoes payload (bring-up / baud validation) |
| 0x06 | READ_LOGITS | empty → 10 × int32 LE |
| 0x07 | DUMP_ACT | layer_id(1) → uint8 activation dump (debug builds only) |
| 0x08 | GET_INFO | empty → contract major/minor, model id, P (parallelism), clock Hz |

### 0.6 Register map (MMIO, for the soft-core path or an equivalent FSM register file)

| Offset | Name | Dir | Meaning |
| --- | --- | --- | --- |
| 0x000 | CTRL | W | bit0 start, bit1 soft reset, bit2 abort |
| 0x004 | STATUS | R | bit0 busy, bit1 done, bit2 error (crc/protocol), bit3 weights loaded |
| 0x008 | RESULT | R | [3:0] argmax digit |
| 0x00C | VERSION | R | 0x0001_0000 = contract 1.0 |
| 0x010 | IMG_WADDR | W | image byte address |
| 0x014 | IMG_WDATA | W | image byte |
| 0x018 | W_WADDR | W | weight byte address (within the layer selected by W_LAYER) |
| 0x01C | W_DATA | W | weight byte |
| 0x020 | W_LAYER | W | layer id for weight writes |
| 0x030 | LOUT_ADDR | W | logit index 0..9 |
| 0x034 | LOUT_DATA | R | int32 logit |

All multi-byte registers are little-endian; IMG_WADDR/W_WADDR auto-increment on write if bit31 is set.

### 0.7 Budgets (both people design to these)

| Quantity | Value | Consequence |
| --- | --- | --- |
| Design clock | 100 MHz (Basys3 W5), 200 MHz only if it closes | timing closure target |
| Parallel MACs P | 16 (v1) → 32 (v2) of 90 DSPs | latency and adder-tree sizing |
| Inference latency | ≤ 1 ms (v1), ≤ 500 µs (v2) | Person B's benchmark target |
| Link throughput | ≥ 1 Mbaud = 2500 B in 25 ms | the system is link-bound, not compute-bound |
| Accuracy | ≥ 98.5% MNIST test (v1), ≥ 98.5% own-drawings (v2) | gates on Person B |
| BRAM | ≤ 40 of 50 blocks total | leaves room for line buffers |

## §1 Artifacts Person A owns

rtl/
  compute/   mac.v, mac_array.v, requant.v, relu.v, pool.v, argmax.v, conv_engine.v, fc_engine.v
  memory/    weight_bram.v, img_buffer.v, line_buffer.v
  io/        uart_rx.v, uart_tx.v, crc8.v, host_if.v
  npu_top.v, top_basys3.v        (+ soc_wrapper.v if you use the R32 soft core)
  hex/       (generated by Person B — read-only for you)
sim/         tb_*.v (structural, by you) ; cocotb tests are Person B's (see §5)
syn/         basys3.xdc, build_basys3.tcl
docs/        utilisation.md, timing.md, latency.md

Never hand-edit anything in hex/. If a weight is wrong, Person B re-exports.

## §2 What Person A must learn

### Tier A1 — Datapath design, not just logic (weeks 1–4)

You already write synthesizable Verilog and have shipped a soft core. What's missing is arithmetic architecture: pipelining a multiply-accumulate, resource sharing, adder trees, and the latency/area tradeoff underneath every choice.

- Steve Kilts, Advanced FPGA Design: Architecture, Implementation, and Optimization — the single most relevant book. Read: pipelining, resource sharing, area/speed tradeoffs, clock domain crossing, coding for synthesis. Why: it is the book about building datapaths; almost every phase H1–H5 decision is in here. https://www.fpgarelated.com/books/27.php

- Uwe Meyer-Baese, Digital Signal Processing with Field Programmable Gate Arrays — chapters on MAC design, pipelining, systolic FIR filters, distributed arithmetic. Why: a 2-D convolution engine is a systolic FIR filter with a different address generator; the accumulator-width and pipeline-stage reasoning transfers directly.

- Pong P. Chu, FPGA Prototyping by Verilog Examples — reference patterns for BRAM, FSM+datapath splitting, and testbench structure. Why: fast, concrete templates rather than theory.

- Clifford Cummings' papers (nonblocking assignments, coding guidelines) — Sunset/Sunburst, http://www.sunburst-design.com/papers/. Why: the cheap way to never debug a sim/synth mismatch again. Your own history proves this matters: a wrong $readmemh path is silent and yields an all-zero ROM.

Exit test: you have a parameterized, pipelined mac.v (3 stages, signed int8 × int8 → int32) with a cocotb testbench, a positive-WNS timing report, and report_utilization showing DSP48E1 inference. You can explain to Person B where every bit of the accumulator width goes.

### Tier A2 — The DSP48E1 and 7-series memory, in detail (weeks 3–4)

Your gains come from using the hardened silicon instead of LUTs, and from knowing exactly what a BRAM block can and cannot be configured as.

- UG479, 7 Series DSP48E1 Slice User Guide — the 25×18 multiplier, the 48-bit accumulator, the pre-adder, pipeline registers, MACC, SIMD modes. Why: the pre-adder and internal accumulator are how you cut LUT pressure on a 35T; also the source of truth for what infers and what doesn't. https://docs.amd.com/r/en-US/ug479-7series-dsp48e1

- UG473, 7 Series FPGAs Memory Resources — RAMB36E1/RAMB18E1, width configurations (1/2/4/9/18/36), simple vs true dual port, cascade, and what "byte-wide" really gives you (4 KiB per block). https://docs.amd.com/r/en-US/ug473-7series-memory-resources

- UG901 Synthesis, UG903 Constraints, UG949 Methodology, UG906 Timing Analysis — read UG901's RAM/ROM inference coding patterns and UG906's timing-report sections. Why: you will read a routed timing report a hundred times; learn to find the critical path from the report instead of guessing.

- Xilinx WP486, Deep Learning with INT8 Optimization on Xilinx Devices — the vendor's account of int8 on DSP48. Short, and it frames why int8 is the natural precision on this silicon. https://docs.amd.com/v/u/en-US/wp486-deep-learning-int8

Exit test: you can state, without the tool, how many DSP48E1 slices a P-wide int8 MAC array uses, what the accumulator width must be for a 3×3×16-tap dot product, and how many 36 Kb blocks a 20 KiB weight set consumes (answer: 5).

### Tier A3 — Accelerator architecture vocabulary (weeks 4–7)

You can build any of these in RTL; the point is choosing the one that fits 90 DSPs and 225 KiB.

- Sze, Chen, Yang, Emer — Efficient Processing of Deep Neural Networks (Synthesis Lectures). Free precursor paper: Efficient Processing of Deep Neural Networks: A Tutorial and Survey, arXiv:1703.09039. Why: dataflows (weight-stationary / output-stationary / row-stationary), loop tiling and unrolling, the memory hierarchy, and the arithmetic-intensity reasoning that decides your architecture.

- Chen et al., Eyeriss (ISCA 2016) and Jouppi et al., TPU (ISCA 2017) — read for the architecture, skim the numbers. Why: the two canonical array designs your Phase H5 sits between.

- Umuroglu et al., FINN (FPGA 2017) + Xilinx/finn docs — Why: how a dataflow accelerator's control and parallelism are structured; also tells you what you're deliberately NOT adopting.

- MIT 6.5940 "TinyML and Efficient Deep Learning Computing", the "Hardware Acceleration" and "Quantization" lectures — https://hanlab.mit.edu/courses. Why: shared vocabulary with Person B; watch these two lectures together so you both use the same words for the same things.

Exit test: for the v2 CNN you can name, with numbers, the dataflow you chose, the tile sizes, how many cycles each layer takes at P=32, and where every byte of weight and activation storage lives.

### Tier A4 — Verification and bring-up discipline (continuous)

- cocotb docs — https://docs.cocotb.org/. You'll review Person B's tests even though they write them.

- Verilator / Icarus — your simulation of record (your own experience: xsim rejects forward-declared identifiers that synthesis and Icarus accept — simulate with Icarus).

- ZipCPU tutorial + blog — https://zipcpu.com/tutorial, and Learning AXI: where to start? https://zipcpu.com/blog/2022/05/07/learning-axi.html. Why: FSM/datapath style, handshake discipline, and the "assertions before waveforms" mindset. Read the AXI piece before you design host_if.v even though you won't use AXI — the valid/ready reasoning is the same.

- Digilent Basys3 reference manual + master XDC — https://digilent.com/reference/programmable-logic/basys-3/reference-manual and https://github.com/Digilent/digilent-xdc. Verified pins for your board: clk W5 (100 MHz), btnC U18, UART RsRx B18 / RsTx A18, LEDs LD0..U15 start at U16,E19,U19,V19,W18,U15,U14,V14,V13,V3. Why: never trust recalled pin numbers — a wrong pin is a silent dead signal.

Exit test: an ILA or a register-file debug path that lets you see the state of a stalled FSM in one build, rather than re-synthesising to add a probe.

### Tier A5 — Optional, only if you want the comparison (post-v2)

- AMD UG1399 Vitis HLS User Guide — https://docs.amd.com/r/en-US/ug1399-vitis-hls. Why: to write the "hand-written RTL vs HLS on the same network" section of the write-up. Not on the critical path.

### Do NOT spend time on

Vitis AI / DPU (Zynq UltraScale+ and Zynq-7000 with DDR only — an Artix-7 pure-PL part is not a supported target), NVDLA (needs a big FPGA + DDR), FINN's supported flow (Zynq PS+DMA; Artix-7 use is community-hacked — see Xilinx/finn discussion #508), float32 anywhere in hardware, or a 16×16 TPU-style systolic array as v1. Build the 16-lane MAC array first; earn the systolic array in H5.

### Reading order

Wk 1–2 Kilts (pipelining, resource sharing) + Cummings; Wk 2–3 UG479 + UG473; Wk 3–4 Meyer-Baese (MAC, systolic FIR) + UG901/UG906; Wk 4–6 Sze survey + the two MIT lectures with Person B; Wk 5–6 Eyeriss + TPU papers + FINN docs. Everything else is lookup.

## §3 Toolchain setup (do this in week 1, before any design work)

- Vivado ML Standard — free, covers Artix-7 up to 100T, no license file. Install the Basys3 board files.

- Batch build flow (never the GUI for regressions): vivado.bat -mode batch -source build_basys3.tcl. Read report_utilization and report_timing_summary output files programmatically each build.

- Icarus + cocotb for simulation; GTKWave for waveforms.

- openFPGALoader or Vivado Hardware Manager for programming; kill stale serial readers (only one process can hold a COM port — a lingering Python reader makes working hardware look dead).

- Repo skeleton with make sim, make synth, make prog, make regress targets from day one.

## §4 Person A implementation plan

### H0 — Skeleton, loopback, and the baud ceiling (week 1)

Goal: prove the toolchain and measure the link before anything is designed around it.

- Repo skeleton; batch TCL build; blink; LED readout of a counter.

- uart_tx.v / uart_rx.v / crc8.v + a loopback FSM implementing OP 0x05.

- Measure your real maximum error-free baud over 10⁷ bytes (FT2232H does up to 12 Mbaud; 1 Mbaud should be clean, validate it).

- Deliverable: docs/latency.md with the measured baud; loopback at the chosen rate.

- Exit: Person B's 40-line host script round-trips 10⁷ bytes with zero CRC errors.

### H1 — MAC, array, requant, against frozen vectors (weeks 2–3)

- mac.v: signed int8×int8→int32, 3 stages, valid/clear, no accumulation on invalid beats.

- mac_array.v: P=16 lanes + registered adder tree; verify the DSP count in the synthesis log.

- requant.v: shift, round-half-away-from-zero, saturate to uint8. Test with the nasty values: max/min int32, exact .5 ties, negative values, saturation both ends.

- relu.v, argmax.v (lowest index on tie), pool.v.

- Deliverable: cocotb regression (written by Person B, run by you) passing 100% on these modules.

- Exit: every module byte-identical to the golden vectors, P=16, positive WNS at 100 MHz.

### H2 — Weight memory and the layer engine (week 4)

- weight_bram.v: $readmemh from rtl/hex/, absolute path via a parameter INIT_FILE. Grep every synthesis log for $readmem data file ... is read successfully — a wrong path is silent and gives you an all-zero ROM, which looks exactly like a math bug.

- fc_engine.v: FC layer as a dot-product loop over BRAM rows, one weight row per cycle, accumulator cleared per output neuron; img_buffer.v for the input image.

- Layer FSM: address generation, valid gating, done/busy, and a per-layer scale/shift taken from the register file (so Person B can retune scales without an RTL change).

- Exit: layer 1 of v1 matches Person B's case_0000.l0.bin byte-for-byte in simulation.

### H3 — Accelerator integration and host interface (week 5)

- npu_top.v: three layer engines chained, argmax, MMIO register file per §0.6.

- host_if.v: UART framer for OP 0x01/0x02/0x03/0x04/0x06/0x08; write into img_buffer and the weight BRAMs; 0x07 DUMP_ACT behind a debug build flag.

- Integration choice: bare RTL host FSM (simplest, recommended for v1) or reuse your R32 RISC-V core as host with the NPU at 0x4000_0000 — you already have that MMIO pattern working on this board.

- Exit: full-network simulation matching all vectors; GET_INFO returns contract 1.0 / model id / P / clock.

### H4 — v1 on the board (weeks 6–7) — first light

- Program, load weights over UART (~51 KiB = 522 ms at 1 Mbaud, once per boot), send an image, read the result. Prediction on LEDs LD3:0 and the 7-seg display; raw logits available over READ_LOGITS.

- Bring-up order that finds bugs fastest: LOOPBACK → GET_INFO → LOAD_WEIGHTS + status bit3 → START + busy → RESULT. Add a visible status register on the LEDs before you need it.

- Exit (this is the v1 project milestone): 20/20 of Person B's drawn digits classify identically to the golden model, on hardware. Log utilisation and measured latency in docs/.

### H5 — The CNN engine at 50×50 (weeks 8–12) — the actual NPU, and the critical path

- line_buffer.v: 3-row circular buffer so one new pixel/cycle feeds a 3×3 window; no im2col in memory.

- conv_engine.v: weight-stationary, Cout filters spread across PEs, weights loaded once per layer, activations streamed; 9 taps is the natural inner dimension.

- Streaming 2×2 maxpool on the conv output; reuse fc_engine.v as an output-stationary tiled FC (1936→10).

- Accumulator width for 3×3×16 taps; then try the DSP48E1 internal accumulator (MACC) and the pre-adder to fold two window terms before the multiply — both are real LUT savings on a 35T.

- Revisit 200 MHz via MMCM only if you are throughput-starved after the 1 Mbaud link is saturated (you probably won't be — the system is link-bound).

- Exit: v2 CNN classifies on hardware, timing closed at the chosen clock, latency ≤ 500 µs, byte-exact against golden vectors.

### H6 — Performance, robustness, and the write-up (weeks 13–14)

- Full regression re-run against the final vectors; utilisation/timing/latency tables into docs/.

- Robustness: framing errors, CRC failure, mid-frame reset, weight reload while idle — error bit must be visible and recoverable, not a silent wedge.

- Optional stretch, only if the core is done: BNN variant (XNOR + popcount in LUTs instead of DSPs) for a comparison table, or a port to Arty A7-100T (same RTL, 2.7× BRAM, 240 DSP) in one TCL change.

## §5 JOINT WORK — sections that require Person A and Person B together

These are not "nice to sync up" items. Each one produces an artifact neither person can produce alone.

| ID | When | What | Deliverable / exit |
| --- | --- | --- | --- |
| J0 | Week 1 | Contract freeze. Walk §0 line by line. Each person independently writes a CRC8 implementation and a frame parser; compare against 5 published test frames. Agree the register map, vector format, numeric spec, and the rule that changes are joint. | CONTRACT.md v1.0 committed, plus a shared protocol_frames.txt both parsers agree on. |
| J1 | Weeks 3–5 | Golden vector handoff. B publishes vectors + the integer model; A runs them in cocotb. Every mismatch is diagnosed together — is it the RTL or the spec? This is where rounding bugs and accumulator-width bugs surface. | MAC/array/requant/layer regression at 100%, and a written note of every numeric discrepancy found and which side it was. |
| J2 | Weeks 6–7 | First-light bring-up session. Both people at the board: A watches LEDs/ILA and reloads bitstreams, B runs the host driver and reports what the wire actually carried. Do not do the first hardware run of a real image alone. | 20/20 drawn digits match the integer model on hardware. |
| J3 | Weeks 8–12, on every numeric change | Change control on the contract. If either person wants different bit widths, a per-channel scale, a new rounding mode, or an extra opcode: both agree, version bumps, B re-exports all vectors and hex, A re-runs the full regression. Nobody edits the contract silently. | Version-stamped CONTRACT.md + regenerated vectors + green regression. |
| J4 | Monthly | Accuracy ↔ throughput tradeoff review. B brings the precision/accuracy table (int8 vs 4-bit vs 1-bit), A brings latency/utilization at each P. Decide together which point on the curve the project targets — this is an architecture decision, not either person's call. | A one-page decision record in docs/tradeoffs.md with the numbers that justified it. |
| J5 | Weeks 13–14 | Demo and write-up. B's GUI/VGA assets and A's bitstream and measured numbers are one artifact. A supplies utilisation/timing/latency; B supplies accuracy/precision tables; both review the final report. | Final demo + report/README with numbers both people can defend. |

Standing rule: a "green" claim requires A's regression log AND B's vector manifest SHA. A bitstream run with mismatched hashes is not evidence.

## §6 Interfaces: what you own vs. what you consume

| Interface | Owner | You must |
| --- | --- | --- |
| rtl/**, syn/** (XDC, TCL), bitstream | A | change freely, but never break register map or frame format |
| hex/*.hex, model.json, vectors/** | B | treat as read-only input; report failures, don't patch |
| sim/cocotb/** | B writes, A runs | review the tests; you own the structural tb_*.v |
| Register map / UART ops / numeric spec (§0.4–0.6) | joint | change only via J3 |
| Contract version + SHA gate | joint | enforce it in your own flows |
| Latency/throughput budget, accuracy target | joint | A is accountable for latency, B for accuracy |

## §7 Things Person A must never do

- Hand-edit a .hex or a vector file to make a test pass.

- Change the register map, an opcode, or a rounding rule without J3.

- Claim a result from a bitstream whose model.json SHA doesn't match the vectors used in simulation.

- Trust a recalled pin number instead of the Digilent master XDC.

- Add a debug probe (sampled PC on LEDs, a mux) and forget to remove it — a restore that silently fails ships the debug display in the next bitstream and misreports the design's state. Grep for the debug signal names and confirm zero references before building.

- Report "it works" from utilisation/timing reports alone; only a hardware classification counts.

## §8 Risks specific to Person A

| Risk | Mitigation |
| --- | --- |
| Silent all-zero weight ROM | grep read successfully every build; assert a non-zero checksum of loaded weights in hardware |
| Accumulating on invalid beats | valid gating + a cocotb test that injects invalid beats mid-accumulation |
| Timing fails at 100 MHz | find the critical path in the routed report before reducing the clock — usually a wide mux or an unregistered BRAM output |
| BRAM over-budget when line buffers appear | budget 40/50 blocks from the start; the CNN needs ~5 for weights, the rest is buffers |
| Two builds disagree (GUI-held project lock) | build into a separate project dir; never delete/recreate a dir the GUI has open |
| Long debug loops | visible status register + UART as the debug channel from H0 onward |

## §9 Schedule (aligned with Person B's document)

| Week | Person A | Person B | Joint |
| --- | --- | --- | --- |
| 1 | H0 skeleton, loopback, baud measurement | S0 environment, contract, host driver stub | J0 |
| 2–3 | H1 mac/array/requant | S1 dataset + own-code trainer |  |
| 4 | H2 weight BRAM + layer engine | S2 quantiser + exporter |  |
| 5 | H3 integration + host_if | S3 integer golden model + vectors | J1 |
| 6–7 | H4 first light on board | S4 host driver + drawing GUI | J2 |
| 8–9 | H5 line buffers + conv engine | S5 own-data collection + retrain |  |
| 10–12 | H5 conv+pool+FC v2, timing closure | S6 benchmark harness, precision study | J4 |
| 13–14 | H6 robustness + number tables | S7 demo assets, report sections | J3, J5 |

Critical path: H5. Person B's work is front-loaded precisely so that A is never blocked waiting for weights or vectors.